Instructions for preliminary work before running “Fibrils_AF647_macro_w_scramble_width_intensity_annotated.ijm”

Making background images

- The first step to this process is to make the ROI folder. To do this open a image (probably easier to open the 488 image for this). Then draw a rectangle around the right hand side channel. Then go to Analyse  Tools  ROI manager and click add. Then within the ROI manager rename this ROI to “0”. Then go back to the image and change the ROI slightly (slightly crop like 1 mm on one side or something) and then click add. Change this ROI’s name to be “1”. Then highlight both of them at once and go to More  Save and save this as “ROI.zip” and put it in the “AF647 photobleach” folder.

- Next step is to make background images for the 488 and 647 channels. Let’s do the 488 (fibril) images first. Open up the images and choose one that looks pretty uniform (i.e. not a super bright blob or fibril in one corner). Once you have selected an image then in the ROI manager selected the ROI that is the biggest (should be 0 every time) and then do image  Duplicate and name this duplicate “fibril_background”. Then do Image  Type  32-bit and then do Process  Filters  Gaussian Blur and set the Sigma (Radius) to 40 and then press OK. Then do File  Save as  Tiff and save it in the “AF647 photobleach” folder where the ROI.zip was saved. Follow the same process to get the background image for the 647 channel with these differences: if you have a photobleaching movie then select the ROI and use the little bar at the bottom of the image to move to the last frame of the movie when everything should be bleached, then duplicate this and this time name it “AF647_background” and process and save the same as the 488 fibril one.

Now we have all of that it is time to run the actual scripts .